# Notes on turning this into a journal paper.

# Optimisation of CDMA-Based Mobile Telephone Networks: Algorithmic Studies on Real-World Networks

Paul Weal1, David Corne2, Chris Murphy3

1SECaM , Harrison Building, University of Exeter, EX4 4QF, UK

P.D.Weal@ex.ac.uk

2MACS, Earl Mountbatten Building, Heriot-Watt University, Edinburgh EH14 8AS, UK
dwcorne@macs.hw.ac.uk

3Motorola Ltd, Thamesdown Drive,  Swindon, Wiltshire, SN25 4XY, UK
Chris.Murphy@motorola.com

Abstract.  CDMA and WCDMA mobile phone networks depend on a network of antennae, each defining a geographic ‘cell’ that handles the transmissions to and from users’ handsets within that cell. These antennae have adjustable settings whose values have a large effect on both quality of service (and consequent subscriptions) and resource consumption. We consider the optimisation of these parameters, and describe experiments that compare a range of optimisation algorithms with the methods currently used in the field for this purpose. The aim of the current project was to achieve faster (necessary) and better (if possible) results than the existing methods used by field engineers. We find that certain evolutionary algorithm configurations achieve both of these requirements on test problems arising from real data from a high-raffic urban environment. To some extent the ideal algorithm depends on the size and load in the network being optimised, and this is the main topic of ongoing research.

- 1   Introduction

We report on a project aimed at improving the dynamic service delivery infrastructure of a mobile phone network provider, via exploring a small number of algorithms for optimising aspects of that infrastructure. We specifically address optimisation of infrastructural elements of a mobile telephone network based on Code Division Multiple Access (CDMA) technology. A mobile telephone communications service provider needs to maintain a network of antennae, each defining a geographic ‘cell’ and responsible for providing transmission services for handsets within (or near) that cell. A cell’s extent must be such that subscribers may move from the coverage area of one cell to another with continuous service. That is, there should be an area of overlap between neighbouring cells. But too much overlap would lead to too much interference, resulting in excessive powers and reduced capacity to support many subscribers. Each antenna will have at least one CDMA downlink radio carrier which is used to transmit signals to mobiles in that cell. There are many parameters associated with each carrier but the most significant for adjusting the geographical relationship between cells is the ‘pilot power’. This is adjustable typically via a database in the central operations and maintenance centre, and relates directly to the strength of signal received by users’ mobile devices when it is choosing its serving cell. We also consider the adjustment of other parameters, which we discuss later.

Motorola has developed a suite of tools for modeling and simulating the interactions between cells given defined user loads and system parameters. Two optimisation methods are in this suite, which, using the simulator to evaluate configurations, attempt to find good configurations of pilot powers for specific periods of the day. These are a proprietary iterative local/greedy search style algorithm (which we call ILS), and a simple evolutionary algorithm (MotGA). Engineers in the field currently use ILS, since it works quickly (within a few hours on modest spec hardware), and gives acceptable results in that time. MotGA is capable of producing better solutions, but currently the time taken to is unacceptable to the field engineers.

The remainder is set out as follows. In section 2 we review some relevant related material. Section 3 then further describes and details certain specifics of the optimisation problem we address and the test data. In section 4 we describe algorithms and experiments and present the results, and we end with a brief discussion in section 5.

2  Related Work

# See if you can find 3 or 4 more papers in each `category’, using scholar.google.com  and ieeexplore – note that I can get you papers from ieeeexplore. Also I can get papers from the ACM Digital library (but so can you – cleck on their `free’ thing and you can sign up).

There are several optimisation issues involved in CDMA mobile telephone networks, but they fall cleanly into three categories, being mainly concerned with issues such as the realtime scheduling of signals at a single base station, the initial design and planning (e.g.. antenna placement) of a new network, or the continual optimisation and management of an established network. As an example of the first category, [1] considers the case of high data rate customers using a network that also carries a large number of voice users; based on a mathematical model, they derive principles and an associated optimal downlink (from antenna to user) signal scheduling algorithm that minimises interference between data and voice users. Another example in the first category is [2], in which an EA is proposed that works separately within each cell; it regularly samples user traffic and attributes at that cell, and the EA determines optimal bit rate and power setting for each mobile user, and transmits that information back to them. In tests this yielded significant quality of service improvement.

Meanwhile, concerning initial design and planning, this is often solved by experienced engineers using simulators with Monte Carlo methods to generate good initial parameterisations. However [3] considers together the location and pilot powers of base stations (antenna), in order to maximise network capacity, based on a mathematical model that enables a mixed integer programming (MIP) formulation, solved via both MIP and adaptive simulated annealing [4]. They addressed small problems (fewer than 30 cells). [5] and [6] consider a similar problem, and address it with tabu search and evolutionary algorithms respectively, achieving (according to the model) close to optimal performance for some large networks; these both relate to initial network design, combined with initial or `typical’ power settings. In our work, the problem is to respond dynamically to existing demand in an already established network.

In practice the third category of continual ‘re’-optimisation of an existing network is traditionally done by teams of engineers performing drive tests around the network, with test mobiles and spectrum scanners for measuring received signal code power of the pilot channels. This identifies problems of poor coverage or interference, and attempts are made to fix them by intuitive tuning of system parameters in the region. There is relatively little published work to date in this category, perhaps owing to a relative perceived difficulty in the logistics of applying this optimisation. Given a revised collection of antenna adjustments for an urban network spanning (say) 100 square miles, engineers need to visit each antenna and make adjustments; this effort is hard to justify unless the predicted benefit is quite significant, which is hard to guarantee since the models used contain many approximations. However, work on such a model [7] together with its use to derive optimisation heuristics, showed, in a major city-wide setting, that the resulting optimized pilot powers led to significant gains over the use of uniform pilot settings, traditional ‘ad hoc’ optimisation, and optimization via drive-through tests. As well as significant improvement in pilot interference, there was also gain in forward link coverage and capacity, allowing a substantial reduction in deployment efforts spent in optimizing pilot powers. Our work builds on [7] in two main ways, by using a recently developed model that considerably improves accuracy, and by exploring evolutionary and similar algorithms for the optimisation process. Meanwhile, with the advent of remotely adjustable antennas it is becoming much simpler to adjust the antenna downtilt, azimuth and beamwidth parameters.

Finally, it should be noted that there is much, but considerably less closely related work in the general field of telecommunications. A sample covering the main application areas (optimisation of various kinds of network design, and optimisation of routing through an established network) using evolutionary computation, includes [8—17]. There are also specialised books and proceedings on this specific area [18—20].

3  The CDMA Network Optimization Problem

A couple of nice diagrams would help this section, to illustrate various points.  Also generally expand or add where you think it might improve clarity. We don’t have the tight length constraints in a journal paper, but we don’t want to go overboard.

In a CDMA network,, there may be several hundred or more antenna in a reasonably large city. Eachdefines a ‘cell’, corresponding to the geographic region around it within which it is the preferred base station for users’ handsets in that area. The overall quality of service experienced by users depends on various attributes of these antennae. Amongst these attributes is the ‘pilot power’, which is essentially the strength of a beacon signal provided by it for mobiles to use when choosing which cell to request a service from. Other adjustable attributes include those concerned with the directional nature of the signal, namely the antenna’s azimuth and  downtilt..

A healthy network will be tuned such that the areas of overlap are minimised without compromising the coverage offered. It will also keep power and hence interference to a minimum by ensuring that mobiles are served by the most optimum cell without allowing certain cells to carry disproportionately high traffic. The desire to minimise interference by keeping power under control leads to a complex optimisation problem in finding a balance between quality of service provision and cost as power for one user is interference to another.

As indicated, ‘pilot power’ is the primary parameter available for adjustment. The second is the downtilt of the antenna; the larger this is, the less area an antenna’s signal will cover. The third is ‘azimuth’, this being the antenna's angle from magnetic north, which is often set in relation to topological restraints..

The specific (real) network used in most of our tests, which we call N1, contains 207 cells, of which 59 cells have fixed pilot powers for operational reasons. The remaining 148 cells can have their pilot powers adjusted between 1.0 watts and 3.0 watts in increments of 0.1 watts. Another real network that appears in some of the tests, which we call N2, contains 97 cells, 22 of which have fixed pilot powers; the remaining 77 can have their powers adjusted in the same way as with N1.

The fitness function is an equally weighted sum of the following. 1: Mean traffic channel power per user; 2: Percentage of users who don't receive a sufficiently strong pilot signal; 3: Mean pilot pollution factor (number of non-dominant pilot signals received within a margin, typically 5dB of the dominant); 4: Mean downlink traffic channel outage (users unable to get a usable signal for voice or data services); 5: Penalties associated with users who, to get a usable signal, demand more power from the Linear Power Amplifier than it can provide; 6: Penalty for users served by a cell other than thatclosest in terms of propagation loss. This arises from having non-uniform pilot powers across the network and takes account of uplink optimization, which may conflict with downlink optimisation which is the focus of this paper.

Naturally this can be treated via multiobjective algorithms [21—23], and such is the topic of ongoing related work, however, for this problem these tend to be substantially slower to optimise a network to the extent that they are of little use currently in operational network optimisation. In the present work there is a need to stay close to the current practice of engineers who would use the system, who are familiar with, and require the speed, of the existing single-objective fitness function.

- Algorithms, Experiments and Results

We need to try `proper’ simulated annealing; what you have done so far is a Monte Carlo method, but not really SA.  In SA, assuming a minimization problem, what happens is this:

Fitness of current solution (i.e. best so far) is Fc.

Mutate it – fitness of mutant is Fm.

Let D be Fc minus Fm – so, if D is positive, it is an improvement, if negative it’s worse

The probability of accepting the mutant as the current solution is:  eD/T

(e to the (D over T))

So when D is 0 or positive, this will always by 1 or more, so always accept. If D is negative, this gives something between 0 and 1. This is lower (so less chance of accepting), the worse Fm is, and the smaller T is

T starts from a large value Tstart, and goes  down to a small value Tend at the end.

Simply let Tend be 0.001

Choose Tstart to be big enough to give a probability of 0.5 for a typical `worse’ move. That is .. do some mutations of random solutions to see what differences in fitness you typically get. Call a typical difference d.

Now solve  e(to the d/Tstart) = 0.5

And set Tstart accordingly

After every eval in SA, the temperature reduces by a ratio r

Given Tstart, Tend, and given the number of evals for the expt (say 10,000), we work out r so that   Tstart times r10,000 = Tend  -- solve that for r.  Then you can get going!

`Proper’ SA is indeed a bit of a pain, since you need to set Tstart for each problem, and reset r whenver you want to run for different amount of time on that problem.  But maybe just half an hour looking at data we have already we’ll give you Tstart for each of our problems.

.

We go on and on about speed vs quality here – but we only really assess quality at the end.  With the existing data/spreadsheets, we can effectively compile the quality results for as if we had set the algorithms for 1000 evals, 5,000 evals, and 10,000 evals, and 50,000 evals.  (but for SA we’d have to run it anew each time).  In the journal version of the paper, we can have separate tables and statistics for the relative performance of the algorithms at each of these timescales.

Algorithms: The in-house CDMA network model (developed by the third author) can with suitable accuracy estimate the fitness function described above. The present work was conducted specifically to see if the speed (in particular) and quality of results provided by the incumbent methods (ILS and MotGA) could be improved by using alternative algorithms, yet under the constraint that these should be relatively parameter-light and straightforward (implementable readily in this and other scenarios by non-expert practitioners). To this end we explored hillclimbing (HC), simulated annealing (SA), particle swarm optimisation (PSO) and an alternative evolutionary algorithm (SSGA). Each is briefly described below.

ILS is relatively fast, but found less effective in terms of solution quality. We can broadly describe ILS as follows. It takes a parameter h which refers to the number of cells that will be involved in a hotspot. For each group of h cells, where those groups are considered in a specific order based on network data, ILS estimates the quality of all configurations of available pilot power levels for cells in the hotspot (clearly this becomes infeasible for moderate h); the best such configuration is retained, and ILS moves on to consider the next hotspot. This process is repeated for a given number of passes, where one pass involves considering all hotspots in the network in turn.

MotGA is a classical, generational genetic algorithm incorporating elitism [24]; it uses binary encoding, single point crossover, bitflip mutation and fitness proportionate selection.  Since the problem is one of minimisation, proportionate selection works with the reciprocal of the fitnesses.

SSGA  is a steady state genetic algorithm [25], using real-number encoding, uniform crossover [26], Gaussian mutation and binary tournament selection. In some experiments we also employ swap mutation (two randomly chosen genes are swapped). In one generation of SSGA, two parents are selected, and one child is generated by uniform crossover, and then mutation is applied to that child, and it is evaluated. If the child’s fitness is better or equal to that of the population’s current worst, then it enters the population ovewriting a worst chromosome; otherwise, the child is discarded.

PSO is a standard implementation of Particle Swarm Optimisation (PSO) [27], with parameters c1and c2both set at 1, and Vmax=1. Some preliminary investigation of alternative parameters was done, but none improved significantly on these.

HC is straightforward stochastic ‘hill climbing’. Maintaining a single chromosome (initially random), called the ‘current’, HC iterates the following process: the current is copied and mutated, producing a mutant. If the mutant is fitter than or equally as fit as the current, then the mutant becomes the new current solution; otherwise the mutant is discarded. HC used the same mutation operators as the SSGA.

SA was a simple implementation of simulated annealing [28, 29] with a linearly reducing temperature schedule which was engineered to change the probability of accepting a worse solution from 0.3 at the beginning of the run to 0 at the end. Limited preliminary parametric investigation was done, confirming this as a valid choice.

Encodings and Operators:  In the cases of HC, PSO, SA and SSGA,  a ‘pilots only’ chromosome comprises L real numbers where L is the number of antennae whose pilot powers are not pre-specified, each representing the pilot power of a specific antenna, and the value is constrained in [1, 3]. Let the value of gene i be  vi; when gene i is interpreted, it indicates a setting of p for the pilot power associated with gene i, where p is the closest value to vi in the set {1.0, 1.1, 1.2, …, 3.0} (the available pilot power settings). In the case of MotGA, the encoding is binary, where each power is encoded in an 8-bit string similarly interpreted in [1, 3]. With ILS, the encoding is immaterial (since there is no interaction with genetic operators). In the cases When we optimise each of pilots, tilts and azimuths (see section 3), a chromosome is of length 3L, each group of three genes denoting pilot power, tilt and azimuth values respectively. Each is interpreted similarly as one of thepermitted vales for the corresponding parameter. The  possible pilot powers are as stated above, while tilts and azimuths each have 20 possible settings

In this version, let’s have material about the diversity analysis. I.e. when we took a typical problem and looked at the best solutions from each of several runs, and compared them. If we can describe this, and show some pictures, then that’s a useful addition.   But can we also investigate Ps, Ts and As with similar thing in mind for this paper?

As well as the Gaussian mutation operator described, the SSGA experiments and some of the HC and SA experiments also used a swap operator. This was tried after observing (in work otherwise not reported here) a high diversity in solutions from different runs (also observed in [7]). Ideal solutions tend to be characterised by a pattern in which low power antenna are compensated for by higher power antennae nearby, which in turn suggests that swap mutation is a reasonable tool to explore this space. When is used, it is in combination with the Gaussian mutation operator: a number k of swaps are done (where k = 0 defaults to Gaussian mutation), followed by application of the Gaussian mutation. In each SA, HC and SSGA case, preliminary experiments explored values of k from 0 to 3. In the results reported here, we use only the thus-found best k for each algorithm/problem combination.

Parameters: Each of MotGA, SSGA and PSO used a population size of 100. Whenever Gaussian mutation was used, it employed a mean of 0 and a standard deviation of 0.3, and was applied with a strength of 3/L (that is, mutation was applied to three randomly chosen genes). These values were arrived at on the basis of preliminary experiments. As was implicit before, crossover and mutation were always applied (i.e. crossover rate and mutation rate were both 1).

Experiments and Results: The fitness function  is computationally highly expensive, so our experiments were oriented towards examining the number of evaluations required to achieve acceptable solutions; this is why we are interested in converngence-over-time curves in the following. In every experiment, the `solution quality’ result indicates the best solution after 100,000 fitness evaluations. First, each algorithm was compared on problem N1, optimising only the pilot powers. Figure 1 shows early convergence plots, while Table 1 summarises a statistical analysis of the results using 10,000-sample Randomisation tests [29] based on 10 trial runs of each algorithm. The same collection of algorithms, but omitting ILS, were compared on problem N2, also optimising only pilot powers. We omit the equivalent of Fig 1 for space reasons, which was highly similar to the convergence curves for N1; however we provide a statistical summary in Table 2. Next we investigated optimising pilots, tilts and azimuths on the larger network N1, using only the algorithms more promising in the previous tests, while keeping MotGA for baseline comparison. As before, the HC and SSGA results provided are for the configuration (number of swaps) that was best in preliminary tests. Figure 2 shows convergence behaviour, and Table 3 summarises the results statistically.

Table 1. Summary of statistical comparison of  SSGA, HC, MotGA, ILS, PSO and SA on N1, pilot powers optimisation. Based on best of 10 runs of 100,000 evaluations for each algorithm.

| Comparison | Conclusions |
| --- | --- |
| SSGA vs HC | SSGA superior, p < .00001; HC superior at short timescales |
| SSGA vs MotGA | p = 0.116 at 100,000 evals, p < 0.00001 at 50,000 evals; SSGA superior at short and medium timescales but superior with only low confidence at 100,000 evals point. |
| MotGA vs HC | MotGA superior with >97% confidence, p = 0.023 |
| Others | At 100,000 evals, each of HC, SSGA,MotGA were superior to the various ILS versions, SA and PSO with p < .00001 |

Basic Observations: In the pilots-only/larger network case, SSGA (1-swap) and HC (0-swap) are superior to theothers in terms of both finding good solutions quickly and final solution quality. Both ILS and MotGA are outperformed significantly. At around the 15,000 evaluations mark (beyond which ILS never improves), both SSGA and HC outperform ILS and MotGA, in  time very acceptable to the field engineers. Relative performance was similar on the smaller network N2, except that the difference between HC and SSGA in early stages was less pronounced. Again, SSGA provided better overall results and both outperformedILS and MotGA over long and short timescales. Interestingly, in both ‘pilots only’ cases, the better version of SSGA (hence the one reported here) was that involving 1 swap before the Gaussian mutation, but the better version of HC was that with Gaussian mutation alone.

Table 2. Summary of statistical comparison of  SSGA, HC, MotGA,  on problem N2, (abbreviated to these for space reasons); pilot powers optimisation. Based on best from 10 runs of 100,000 evaluations for each algorithm  .

| Comparison | Conclusion | p-value |
| --- | --- | --- |
| SSGA vs HC | SSGA superior, >99% confidence | 0.0016 |
| SSGA vs MotGA | SSGA is superior | < 0.00001 |
| HC vs MotGA | HC superior, >99% confidence | 0.0014 |

Table 3. Summary of statistical comparisons of SSGA, HC, MotGA, on problem N1; pilot powers, tilts and azimuths optimisation. Based on best of 10 runs of 100,000 evaluations.

| Comparison | Conclusion | p-value |
| --- | --- | --- |
| HC vs SSGA | Inconclusive | 0.36 |
| HC vs MotGA | HC is superior | < 0.00001 |
| SSGA vs MotGA | SSGA is superior | < 0.00001 |

10 more runs of HC vs SSGA please to see if we can get a conclusive result.

Concerning the ‘pilots, tilts and azimuths’ problem on N1, it is notable that in very early stages the chromosomes are much less fit than in the corresponding stages of the ‘pilot powers only’ runs. This arises because tilts and azimuths are randomly assigned in the initial population, rather than given a predetermined reasonable assignment in the pilots-only case. However, being able to adjust these extra parameters clearly leads to benefits, with cost falling to around 27 in these experiments rather than 30.  Again, HC and SSGA both outperform the incumbent MotGA. Meanwhile, ILS is inapplicable in the ‘pilots, tilts and azimuths’ case; if adapted in the natural way, combinatorial explosion would allow few if any passes through the algorithm, while applying it to pilots optimization alone yields the results already shown in Figure 1. Finally, it is notable that swap mutation was not useful in this case; both the SSGA and HC use Gaussian mutation alone.

We can have room for more/better plots. But not sure what’s best at the moment.

Journal version *should* have more expts and things in it than a conference paper -- although this doesn’t matter if the PPSN paper is not accepted. But, anyway, the `proper SA’ runs will be more such expts.  The diversity stuff is also more material.

- Concluding Discussion

Mobile phone networks based on CDMA technology present a plethora of challenges for service provision and maintenance, and these in turn demand sophisticated computational models, up-to-date optmisation approaches and much empirical investigation to address. Also, in common with a pattern typical in the commercial world, new developments in engineering and technology yield even more opportunities and challenges. Hence, as it becomes possible to routinely and easily adjust more attributes of base station antenna, the challenge of optimising those parameters comes to the fore.

Fig. 1.  Early convergence behaviour  on network N1, optimising pilot powers only.  Each curve is mean of ten runs; upper plot shows first 30,000 evaluations of a 100,000 evaluation run, so that we can highlight behaviour early in the run; lower plot provides the results for ILS with different levels of k, contrasted with SSGA and HC, for approx. first 15000 evaluations. None of the ILS runs improved beyond the best level in this plot.

Fig. 2. Convergence behaviour of SSG, HC, and MotGA on network N2 (77 adjustable antennae), optimising pilot powers, tilts and azimuths.  Each curve is the mean of ten runs.

We have looked at the optimisation of pilot powers in CDMA networks, based around using Motorola’s current CDMA modelling package, and also looked at the problem of optimising each of pilots, tilts and azimuths. Our test data came from two established networks, and we were able to find improvements to the incumbent optimisation methods, that are able to find better solutions, and more quickly. On the basis of these results, an ongoing project will deploy the updated methods in the field at a selection of regions. However, this study has been necessarily limited by the computational expense of the fitness function (a single trial run of 100,000 evaluations on network N1 typically takes 24 hrs on a modest spec workstation), and by the fact that it has been difficult so far to obtain additional network data beyond the two reported on here; meanwhile much more needs to be done to understand how suitably to choose the algorithm based on the (ideally) easily measurable details of the network under study and its user load. In our view it is more urgent, however, to find a way to significantly accelerate the speed of experiments on these problems. We are currently addressing this issue via using these CDMA optimisation problems as a case study/research vehicle for landscape state machines (LSMs) [30, 31]. The idea here is that data from previous runs is used to build an approximate landscape model, which can then be used for ultra-fast approximate performance evaluation via running algorithms on the model. Meanwhile the LSMs themselves may be visualised and so yield some further insight into the relationships between problem and algorithm choice.

- References

[1] Bedakar, A., Borst, S., Ramanan, K., Whiting, P. and Yeh, E. (1999), Downlink scheduling in CDMA data networks, in Proc. Global Telecomms. Conf., 1999. GLOBECOM'99, vol 5, pp. 2653—2657.

[2] Moustafa, M., Habib, I., Naghshineh, M. (2001)  Wireless resource management using genetic algorithm for mobiles equilibrium, Proc.6th IEEE Symp. on Computers and Comms., pp. 586—591.

[3] Akl, R.G., Hegde, M.V., Naraghi-Pour, M. and Min, P.S.  (2001)  Multicell CDMA network design, IEEE Transactions on Vehicular Technology, 50(3):711—722.

[4] Chen, S. and Luk, B.L. (1999) Adaptive simulated annealing for optimization in signal processing applications, Signal Processing, 79(1): 117—128.

[5] Lee, C.Y. and Kang, H.G. (2000) Cell planning with capacity expansion in mobile communications: atabu search approach, IEEE Transactions on Vehicular Technology, 49(5):1678—1691.

[6] Maple, C., Guo, L. and Zhang, J. (2004) Parallel Genetic Algs. for Third Generation Mobile Network Planning, Proc Int’l. Conf. on Parallel Comp. in Elec. Eng. (PARELEC'04)   pp. 229—236.

[7] Love, R.T. Beshir, K.A. Schaeffer, D. andNikides, R.S. (1999), A pilot optimization technique for CDMA cellular systems, in Proc. 50th IEEE VTS Vehicular Technology Conf., vol. 4, pp. 2238—2242.

[8] Sinclair, M. (1993) The application of a genetic algorithm to trunk network routing table optimisation, in Proc. 10th UK Teletraffic Symp.: Performance Engineering in Telecomms. Networks, 2/1—2/6.

[9] Abuali, F.N., Schoenefeld, D. and Wainwright, R.L. (1994) Designing telecommunications networks using genetic algorithms and probabilistic minimum spanning trees, in Proceedings of the 1994 ACM symposium on Applied computing, pp. 242—246.

[10] Celli, G., Costamagna, E. and Fanni, A. (1995) Genetic algorithms for telecommunication network optimization, in Proc. IEEE Int’l Conf. on Systems, Man and Cybernetics, vol 2, pp. 1227—1232.

[11] Kumar, A., Pathak, R.M. and Gupta, Y.P. (1995) Genetic-algorithm-based reliability optimization for computer network expansion, IEEE Transactions on Reliability, 44(1).

[12] Ko, K-T., Tang, K-S., Chan, C-Y., Man, K-F. and Kwong, S. (1997) Using genetic algorithms to design mesh networks, COMPUTER, 30(8):56—61.

[13] Webb, A., Turton, B.C.H. and Brown, J.M. (1998) Application of genetic algorithm to a network optimisation problem, in Proc. 6th IEE Conference on Telecommunications, pp. 62—66.

[14] Knowles, J., Oates, M. and Corne, D. (2000) Advanced Multi-Objective Evolutionary Algorithms Applied to Two Problems in Telecommunications, BT Technology Journal, 18(4).

[15] Hsinghua C., Premkumar, G. and Chao-Hsien, C. (2001), Genetic algs. for commus. network design - an empirical study of the factors that influence performance, IEEE Trans. E.C., 5(3):236—249.

[16] Weicker, N., Szabo, G., Weicker, K, Widmayer, P. (2003) Evolutionary multiobjective optimization for base station transmitter placement with frequency assignment, IEEE Trans. E.C.., 7(2):189—203

[17]  Crepu, J.-C., Koukam, A., Lissajoux, T., Caminada, A. (2005) Automatic mesh generation for mobile network dimensioning using evolutionary approach, IEEE Trans Evol. Comp., 9(1):18—30.

[18] Corne, D., Smith, G.D., Oates, M. (eds.) (2000) Telecommunications Optimization: Heuristic and Adaptive Techniques, John Wiley & Sons, 416pp.

[19] Pedrycz, W. (ed.) (2000) Computational Intelligence in Telecommuns. Networks, CRC Press, 450pp.

[20] Wang, L. (ed.) (2003) Soft Computing in Communications, Springer, 2003, 408pp.

[21] Deb, K. (2001) Multiobjective Optimization using Evolutionary Algorithms, Wiley, 518pp.

[22] Corne, D., Deb, K., Fleming, P. and Knowles, J. (2003) The good of the many outweighs the good of the one: evolutionary multiobjective optimization, coNNectionS, 1(1), pp9-13, IEEE Neur. Net. Soc.)

[23] Collete, Y., Siarry, P. (2004) Multiobjective optimization: principles & case studies, Springer, 293pp.

[24] Goldberg, D. (1989), Genetic algs. in search, optimization and machine learning, Addison Wesley..

[25] Syswerda, G. (1991) A study of reproduction in steady state genetic algorithms, FOGA I, Morgan Kaufmann, pp. 94—101.

[26] Syswerda, G. (1989) Uniform crossover in genetic algorithms, Proc. 3rd Int’l Conf. on Genetic Algorithms, pp. 2—9, Morgan Kaufmann.

[27] Kennedy, J., and Eberhart, R. C. (1995). Particle Swarm Optimization, Proc. IEEE International Conference on Neural Networks , IEEE Service Center, 1942-1948.

[28] Kirkpatrick, S., Gelatt, C.D, Vecchi, M.P. (1983), Science, 220(4598) : 671—680.

[29]  Edgington, E.S. (1995) Randomization Tests, Marcel Dekker, 424pp.

[30] Corne, D., Oates, M., Kell, D. (2003) Landscape state machines: tools for evolutionary algorithm performance analyses and landscape /algorithm mapping,. EvoCop ‘03, Springer LNCS, pp. 187—198.

[31] Rowe, W., Corne, D., Knowles, J. (2006) Predicting Stochastic Search Algorithm Performance using Landscape State Machines, Proc. IEEE Con. Evol. Comp. 2006, to appear.