# Evolving Recurrent Neural Networks for Fault Prediction in Refrigeration

…

Figure 9:  Predicting the temperature of a  “faulty” cabinet  [We need to have numbers along the x axis to help discern how useful this is.]

6.  Visualisation and Input Importance Analysis

There are two principle principal motivations for visualising neural networks.  The first of these is to use visualisation as a tool for optimising the architecture of the network: deciding on the correct number of hidden nodes [unclear how vis can help with this] or removing weights which have little affect  effect  on the network’s behaviour [this one ok] .  The second is to use visualisation to “open the black box”: to gain an understanding of the internal representations which are created during training, to select appropriate input variables or to gain an insight into the causal relationships which exist between input and output variables in the underlying problem domain.

In our work, we seek to use visualisation to reinforce lend further insight to [`reinforce’ suggests we’ve made our mind up and are just looking for further evidence to support it] experimental findings regarding network topology and also to help us to understand the effects of input variables on network behaviour.

### 6.1.  Visualising Neural Networks

Various techniques exist for the display of neural network architecture and also for the visualisation of a particular network’s response to different input values.  One of the earliest and probably the best known visualisation technique is the Hinton diagram [v1], which uses black or white squares of varying sizes to show the sign and magnitude of weights values.  Although they give a good indication of weight values, Hinton diagrams provide a somewhat abstract representation of network structure [need to be less vague about what the problem is here – is it hard to relate the weights to where they are in the actual network, or something like that?] .    Bond diagrams [v2] employ a topographical representation of the network which gives a clearer indication of how neurons are connected.  Triangles represent weights; black triangles correspond to positive weights and grey to negative weights; triangle size is proportional to weight magnitude.  Figure 1 shows examples of both Hinton and bond diagrams. [are these visualisations of exactly the same network? Useful to say so if so]

More recently, the triangular connections used in bond diagrams have been replaced by simple lines [v3,v4].  Colour is used along with line thickness to show weight magnitude and sign, while preserving the topological layout introduced in bond diagrams.  These diagrams are often referred to as neural interpretation diagrams [cite two or three who use this term] .

Using the simple visualisation techniques mentioned so far it is possible to deduce some basic facts about a neural network and its inputs, outputs and internal representations [v4].  For example, we can conclude that hidden neurons with low magnitude weights connecting to the output layer do not have a great effect on the overall behaviour of the network.  In simple feed-forward three layer networks we can make judgements on the overall excitory  excitatory (??) or inhibitory nature of an input variable using the input-hidden layer link and hidden-output layer link.  If input-hidden and hidden-output weights share the same sign then the input is excitory sp? (has a positive influence on network output); if the signs differ then the input value is inhibitory.  It is important to note that in order to make any realistic judgements of this type, the network must be trained to the global minimum error.  [I don’t think that follows – this is about judging how a particular network does what it does. I guess you are thinking of judging what role a particular input has in the problem under study. The trouble there is that you could have different networks that are both perfect in accuracy, but might do it in different ways.]

Figure 1.  Hinton diagram (left) showing weights connecting a layer of three neurons (y axis) to a layer of four neurons (x axis) and bond diagram (right) showing connections within a simple neural network.

Due to the distributed nature of concept representations within trained neural networks [v1] it is hard to use basic visualisation techniques to infer much about the underlying problem domain.  Investigation has been performed into the extraction of meaningful information on how a neural network solves a given problem by constructing decision trees [v5], showing decision boundaries as hyperplanes [v6] or plotting hidden neuron responses within a hypercube [v7,8].  Garson’s algorithm [v9] pays specific attention to the importance of input variables by assigning an importance value to each node within a neural network.  The algorithm was later extended by Goh [v10] and Tzeng and Ma [v11].  Garson’s original algorithm multiplies weight magnitudes between layers to derive neuron importance.  Tzeng and Ma’s extended version performs a similar operation to calculate importance, while taking into account input data values and the signs of weights.  All of these techniques focus on non-recurrent three layer (input, hidden, output) neural networks. [good stuff nicely summarised]

The use of neural network visualisation is especially prevalent in the natural sciences [v13–v16], where gaining an understanding of the underlying problem domain is at least as important as creating an accurate neural network model.  [this para cries out to be finished like this:  “E.g. Smith and Wesson used an NN to do X, and their visualisation helpe dthem determine that Y was important and Z was redundant, etc.]

### 6.2.  Simple Weight Based Visualisation

This seems to fly in from nowhere. Suggest introduce in this kind of way. “We now describe a visualisation technique that we have developed for use in the domain of this paper. It combines  notions from the literature concerning weight-based and importance-based visualisation, with adaptations for recurrent networks. We first describe an earlier simple version, and use this to motivate certain design decisions of our current technique which is then described in section 6.3.”

Figure 1 [let’s be careful about figure numbering, when all’s said and done]  shows a recurrent neural network with four inputs and one output.  Neurons are represented by circles and are connected together by lines which represent weights.  Weights connect from left to right, except in the case of recurrent loops.  The top left neuron is a bias unit (B0) which outputs a constant value of 1, it is easier to visualise an external bias than an internal one, since the weights which connect hidden neurons to the bias unit are functionally the same as all other weights in the network.  The neurons below the bias unit are the network inputs (I0 … I3).  The neuron on the far right is an output unit (Q0).  Output neurons have no activation function and are used as placeholders, from which the network’s output can be read.  The output has a single incoming connection from the neuron before it, which has a fixed weight value of 1.  The remainder of the neurons are hidden units (H0 … H4), which have a sigmoid activation function and are arranged in two layers.  The bottom neuron in the left hand layer is a recurrent node (H5).

Weights are visualised by sign and magnitude.  The colour of the line is dependant …ent on the magnitude of the weight; the higher the magnitude the darker the line.  Negative weights are drawn as dotted lines.

In an attempt to show the importance of each neuron in the network, we make the radius of each neuron dependant …ent on the sum of the magnitudes of its outgoing weights.  We expect a neuron to have more effect on the overall activation of the network if its outgoing weights have higher magnitudes.

Figure 1:  Simple network visualisation scheme.  Typical network with 4x5x1x1 topology, trained for 15 minute prediction period.

Inputs I0 and I1 have higher  larger  radii than other inputs in figure 1, which implies that the variables with which they are associated (air on and air off temperatures) are of greater importance to the network.  There is also a clear chain of negative weights connecting from I0 via H1 to the output, which tells us that I0 has an excitory  inhibitory???? effect on the network.

There is, however, a problem with this visualisation technique:  The large, negative weight connecting I2 to H3 (WI2,H3) is a good example of this.  Because the weight in question has a high magnitude, we assume that I2 has a higher importance.  However, since H3 has very low outgoing weights, its output will have only a small impact on the activation of the network and so the large weight connecting I2 to H3 does not actually imply that I2 is important.  In this case, other connections from I2 do suggest that it has some importance to the overall behaviour of the network but, arguably,  we should disregard WI2,H3.

### 6.3.  Importance-Based Visualisation

Figure 2 shows the same network as figure 1 but we use a more advanced visualisation scheme which we hope will is designed to eliminate the problem detailed above.  The technique is similar to those presented in [v9], [v10] and [v11] but is extended to work with recurrent networks.  All output neurons are assigned an importance value of 1.  Importance values for all other neurons are a function of outgoing weight magnitude and the importance of the neuron to which that weight connects.  The function used to calculate the importance In of a non-output neuron, n,  is shown in equation 1.

In=mWnmImiWim	[1]

Where n is the current neuron, m ranges across all neurons to which n has outgoing connections and i ranges across all neurons which have outgoing connections to m (including n).  Wnm, is the magnitude of the weight connecting neuron n to neuron m.

Figure 2:  Importance based network visualisation scheme showing same network as figure X (4x5x1x1, fifteen minute data).

Before the network is drawn, each neuron will be assigned an importance value (such that 0 ≤ In ≤ 1), starting at the outputs propagating back towards the input and bias units.  The process of calculating neuron importance is repeated several times (ten in this case) to ensure that the importance of recurrent neurons is correctly calculated.  The radius of each neuron is dependant on its importance value.  Since output neurons have a fixed importance of 1, they have the largest radius.

Figure 2 shows us that I1 is the most important to the network  (at least, according to equation 1) , while hidden unit H3 is virtually unused.  Because representations are randomly distributed across the various neurons in the network, different training runs result in very different hidden node importance values.  However, since input  neurons are always associated with a given input variable, we are able to take the mean importance for each input across several trained networks and analyse the importance of each input variable. [you da man!]

This leads to interesting results, in this case.  Figure X and figure X show the importance values for each of the four network inputs (air on, air off, refrigerating and defrost) given different prediction periods for the largest and smallest network architectures investigated here.  For short prediction periods (1, 2 and 5 minutes) we see that the two temperature values, especially air off, have higher importance values.  At longer prediction periods the mode inputs become more important.  This corresponds to what we might expect, since at shorter prediction times the “same as now” solution works quite well, while at longer prediction times the delayed mode inputs become more useful to the network.  [Nicer if we can see – Broadly speaking, the results corespond with what we might have expected (blah, as above) but the results also provide new insight that would have been difficult or impossible to predict … [e.g. why is air off more important than air on? Why refr more important than defr? ]  Should also say something about the different architectures, but I’m not sure what:-/.

If only something could be said/done about the hiddens. What about this, for 4x5x5x1 networks, for ex.

Represent a hidden node thus:  e.g.  E-E-I-E   I     means that there was excitatory connection from I0, excite from I1, inhib from I2, and excite from I3, and the connection to the output was inhibitory.   Over 10 or whatever runs, count all of the hiddens that end up in each of the 32 possible types. E.g. you may get:

EEEEE 3

EEEEI  4

EEEIE 7

…

EIEIE 26

…

and conclude that having the EIEI:E pattern seems to be an important feature.

Has nothing like this been done in your trawl of network visualisation world?

Figure X:  Input importance values for various prediction times.  Smallest network architecture (4x5x1x1).

Figure X:  Input importance values for various prediction times.  Smallest network architecture (4x10x8x1).

7.  Concluding Discussion

We show evidence that evolved RNNs are an appropriate technology for the advance prediction of cabinet temperatures and fault conditions in supermarket refrigeration systems. Promising error rates have been achieved for both healthy and ‘fault’ data. These error rates were achieved using only small training datasets and we feel that future work with larger datasets will enable better results and further-ahead prediction, especially when dealing with unseen fault conditions.

Prediction accuracies 15 minutes ahead is particularly interesting. Although later prediction windows (see figure 4) provide low error, we are confused by the dip between 30 minutes and 60 minutes windows, and will investigate these windows further when we understand this. Meanwhile, 15-minute-ahead prediction was capable of distinguishing between normal and healthy operation, and provides enough advance warning to (for example) fix a simple icing-up problem in-store, improving those food items’ temperature records and perhaps saving much cost in loss from that cabinet

References

1.	D. Taylor, D. Corne, D.W. Taylor and J. Harkness "Predicting Alarms in Supermarket Refrigeration Systems Using Evolved Neural Networks and Evolved Rulesets", World Congress on Computational Intelligence (WCCI-2002), Proceedings, IEEE Press (2002)

2.	D. Taylor and D. Corne "Refrigerant Leak Prediction in Supermarkets Using Evolved Neural Networks", Proc. 4th Asia Pacific Conference on Simulated Evolution and Learning (SEAL), (2002)

3.	J. L Elman "Finding Structure in Time", Cognitive Science, (1990)

4.	G. Dorffner "Neural Networks for Time Series Processing", Neural Network World, (1996)

5.	T. Koskela, M. Lehtokangas, J. Saarinen and K. Kaski "Time Series Prediction with Multilayer Perceptron, FIR and Elman Neural Networks", Proc. World Congress on Neural Networks, INNS Press (1996)

6.	T. J. Cholewo and J. M. Zurada "Sequential Network Construction for Time Series Prediction" (1997)

7. C Lee Giles, S. Lawrence and A. C. Tsoi "Noisy Time Series Prediction Using a Recurrent Neural Network and Grammatical Inference", Machine Learning, Springer (2001)

8.	Michael Husken and Peter Stagge "Recurrent Neural Networks for Time Series Classification", Neurocomputing, Elsevier (2003)

9. Y. Bengio, P. Simard and P. Frasconi "Learning Long Term Dependencies with Gradient Descent is Difficult", IEEE Transactions on Neural Networks, IEEE Press (1994)

10. Richard K Belew, John McInerney and Nicol N Schraudolph "Evolving Networks: Using the Genetic Algorithm with Connectionist Learning" (1990)

11. Xin Yao and Yong Liu "A New Evolutionary System For Evolving Artificial Neural Networks", IEEE Transactions on Neural Networks, IEEE Press (1995)

12. Y. Liu, X. Yao "A Population-Based Learning Algorithm Which Learns Both Architectures and Weights of Neural Networks", Chinese Journal of Advanced Software Research, (1996)

13. Xin Yao "Evolving Artificial Neural Networks", Proceedings of the IEEE, IEEE Press (1999)

14. J. D. Knowles and D. Corne "Evolving Neural Networks for Cancer Radiotherapy", Practical Handbook of Genetic Algorithms: Applications, 2nd Edition, Chapman Hall (2000)

15. M. N. Dailey, G. W. Cottrell, C. Padgett and R. Adolphs "EMPATH: A Neural Network that Categorizes Facial Expressions", Journal of Cognitive Neuroscience, (2002)

16. Ajith Abraham "Artificial Neural Networks", Handbook of Measuring System Design, Wiley, (2005)

17. E. Edgington. Randomization Tests. Marcel Dekker, New York, NY, 1995.

v1. G E Hinton, J L McClelland, D E Rumelhart, "Distributed Representations" (1984)

v2. Jakub Wejchert, Gerald Tesauro, "Neural Network Visualization" (1989)

v3. Matthew J Streeter, Matthew O Ward, Sergio A Alvarez, "NVIS: An interactive visualization tool for neural networks" Visual Data Exploration and Analysis (2001)

v4. J D Olden, Donald A Jackson, "Illuminating the ‘‘black box’’: a randomization approach for understanding variable contributions in artificial neural networks" Ecological Modelling, Elsevier (2002)

v5. Mark W Craven, Jude W Shavlik, "Extracting Comprehensible Concept Representations from Trained Neural Networks" IJCAI Workshop on Comprehensibility in Machine Learning (1995)

v6. Lori Pratt, Steve Nicodemus, "Case Studies in the Use of a Hyperplane Animator for Neural Network Research" World Congress on Computational Intelligence (WCCI 1994), IEEE Press (1994)

v7. Wlodzislaw Duch, "Visualization of Hidden Node Activity in Neural Networks: I. Visualization Methods" ICAISC 2004 Artificial Intelligence and Soft Computing, Springer (2004)

v8. Wlodzislaw Duch, "Visualization of Hidden Node Activity in Neural Networks: II. Application to RBF Networks" ICAISC 2004 Artificial Intelligence and Soft Computing, Springer (2004)

v9. G. David Garson, "Interpreting neural-network connection weights" AI Expert, Miller Freeman, Inc (1991)

v10. A T C Goh, "Backpropagation Neural Networks for Modeling Complex-Systems" Artificial Intelligence in Engineering, Elsevier (1995)

v11. Fan Yin Tzeng, Kwan Liu Ma, "Opening the Black Box - Data Driven Visualization of Neural Networks" (2005)

v12. A H Sung, "Ranking Importance of Input Parameters Of Neural Networks" Expert Systems with Applications, Elsevier (1998)

v13. D G Chen, D M Ware, "A neural network model for forecasting fish stock recruitment" Canadian Journal of Fisheries and Aquatic Sciences, NRC (Canada) Press (1999)

v14. Michele Scardi, Lawrence W Harding, "Developing an empirical model of phytoplankton primary production: a neural network case study" Ecological Modelling, Elsevier (1999)

v15. Ioannis Dimopoulos, J Chronopoulos, A Chronopoulou-Sereli, Sovan Lek, "Neural network models to study relationships between lead concentration in grasses and permanent urban descriptors in Athens city (Greece)" Ecological Modelling, Elsevier (1999)

v16. H R Maier, G C Dandy, M D Burch, "Use of artificial neural networks for modelling cyanobacteria Anabaena spp. in the River Murray, South Australia" Ecological Modelling, Elsevier (1998)